_____________________________________

Solutions for Convergence OT and IT, Implementation of Enterprise-Level information contextualization from Industrial Data sources.

__________________________________________

Contents

| Tools and prerequisites 2 |  |
| --- | --- |
| Exploring OT/IT convergence solutions 3 |  |
| Developing OT/IT convergence solutions |  |
| Data Sources 5 |  |
| Models 7 |  |
| Use case 1: Smart Objects 12 |  |
| Use Case 2: FactoryTalk Live Data Shortcut 15 |  |
| FactoryTalk Edge Gateway Applications |  |
| Test Client Application 22 |  |
| ThingWorx Application 23 |  |
| SQL Server Application 30 |  |
| Azure IoT Hub Application 37 |  |
| Back up FactoryTalk Edge Gateway configuration 43 |  |
| Restore FactoryTalk Edge Gateway configuration 44 |  |

Tools & prerequisites

Software:

_ Studio 5000 Logix Emulate v33

_ Studio 5000 Logix Designer v34

_ FactoryTalk Logix Echo v33.11.00

_ Studio 5000 Application Code Manager v4.02.00

_ FactoryTalk ViewME v12.00.00

_ FactoryTalk Edge Gateway v3.00.00

_ ThingWorx 9.0

_ FactoryTalk Administration Console v6.30.00

_ Microsoft SQL Server Management Studio v18.9.1

_ SQLServer/SQLExpress

_ Azure IoT Explorer Preview v15.00

_ Azure IoT Hub

Hardware:

_ CompactGuardLogix (1769-L36ERMS) FW v34

_ PowerFlex 525 110VAC, 0.5 H.P. (25B-V2P5N104)

Files:

ACMTR88HMI.apa

FTEGSmartObjects.acd

PACL36ERMS.acd

Exploring OT/IT convergence solutions

Target is to organize large volume of high-speed data with OT-based Information models. In high-speed production scenarios, it’s common for manufacturers the need to quickly access the entire set of operational related data (production, quality, maintenance, etc.) from various stages in the process.

Approach

An accurate aggregation of plant floor related data with timestamp is critical for implementing analytics, contextualize and generating customized reports for decision making to optimize production processes. Manufacturers can build a common information model that contains the proper data structure at each stage. This information model is built at the data source and can be pushed to data storage or IT applications – either on premises or in the cloud.

Solution interface: FactoryTalk Edge Gateway software

FactoryTalk Edge Gateway software unifies data from industrial sources and control or automation systems. It integrates with a variety of cloud, IIoT, and big data applications including ThingWorx, Microsoft Azure IoT Hub, Microsoft SQL and more. This maximizes operational insights, simplifying and automating data ingestion in a single integration solution for IT applications.

With FactoryTalk Edge Gateway software user can create information models at the Operational Technology (OT) layer and efficiently map to upstream IT applications to improve performance KPIs like OEE, predictive quality and process efficiency.

With access to OT contextualized data, IT applications can help users to extract equipment insights across the enterprise. Quickly, easily, and proactively identify process and production

inefficiencies and product defects. With less required data preparation time, customer can focus on using their data to detect and resolve equipment issues and improve production output.

Capabilities/Benefits

- Simplifies and automates industrial data collection, contextualization, and organization across equipment, devices, and automation assets

- Enables IT/OT convergence

- Unlocks key industrial performance insights from OT data

- Enables high IT data integrity

- Integrates with a variety of cloud, IIoT and big data applications

- High speed, synchronous data collection

- Reduces the need for IT technical expertise

Driving industrial connectivity through FactoryTalk Edge Gateway software

Supports OT / IT collaboration by empowering control engineers -Without traditional IT skills- to identify the most appropriate data sets (Information models) and send information directly to the IT layer.

Application level modular scope:

- Connect to data sources – these could be Rockwell or 3rd party Ethernet/IP (Drive & IMC, Logix v16+, FactoryTalk Smart Object Information Models and data), FactoryTalk Live Data.

- Contextualize the data into Information Models: Allows OT engineer to organize data in context as it pertains to the OT assets, simplifies maintenance and updates.

- Map the data to enterprise and cloud applications: Supports Azure IoT Hub and Azure IoT Edge, Creates SQL table needed to store data and creates objects in ThingWorx to replicate the gateway information model.

Developing OT/IT convergence solutions

Data sources

From Data Sources, first define drivers and then configure data sources. Drivers define the communication method used by FactoryTalk Edge Gateway to collect data from data sources.

Configure data sources to collect data from data tags, models, and FactoryTalk Smart Object instances. FactoryTalk Smart Object instances require an offline Studio 5000 Logix Designer project file (.acd) and an EtherNet/IP driver for data collection.

Drivers enable communication between the gateway and data sources. Add drivers to configure data collection paths for data sources and FactoryTalk Smart Object instances. This guide covers following data sources configuration:

- Add a data source from a Studio 5000 Logix Designer Project File (.acd)

- Add a data source from a Studio 5000 Logix Designer Controller via EtherNet/IP

- Add a data source from a FactoryTalk Live Data Shortcut

Go Online to add a driver. By default, an EtherNet/IP (CIP) driver is installed with FactoryTalk Edge Gateway.

To add a driver:

1. Select Data Flow > Data Sources.

2. From Define Drivers, select Add to add a driver to the gateway.

3. From Select Driver, select the driver type to configure:

• FactoryTalk Live Data (FTLD)

• EtherNet/IP (CIP)

4. Select Next.

5. Enter the Driver Name and define the settings.

• The name must contain only letters, numbers, and hyphens.

• The settings differ for each driver type.

6. Select Save.

FactoryTalk Edge Gateway gathers the directory of data from the source and stores the data into a namespace. In the case of FactoryTalk Smart Object instances, the FactoryTalk Smart Object model is also discovered and replicated in Models. Use these data tags and models to configure the gateway.

From Configure Data Sources, add, edit, delete, populate, clear, and filter data sources.

Models

Use models to organize data into a hierarchy. Associate models with applications or data destinations.

Build models by staging tags, and then associating the tags to a node or property. Use parent and child nodes to form a hierarchy of information elements.

These are guidelines for models:

• There is a maximum of 30 models per gateway.

• There is a maximum of 500 parent nodes per model.

• There is a maximum of eight layers per model.

To add a model

1. Go to Data Flow > Models.

2. Select Add.

3. Type a name for the model and then select Save.

The new model appears in the Models table.

To add nodes or properties to a model

1. Perform one of these actions:

• Select Data Flow > Models.

• Select Data Flow and then under Organize, select Configure.

2. Select Edit next to the model.

3. Under Models, select Add Node.

4. Type a name for the root node and press Enter.

5. To add a child node, right-click the parent node, select Add Child, and then select Node.

6. To add a child property under a node, right-click the parent node, select Add Child, and then select Property.

7. Type the name of the child node or property and then press Enter.

8. To change the hierarchy, drag and then drop the nodes or properties.

9. To move a node to the highest level, right-click the node and then select Move to Root.

10. To rename a node or property, right-click the node or property and then select Rename. Type a new name.

Staging a tag involves moving content from a namespace to the Staging Area and then mapping tags to a model. Once tags are in the Staging Area, bind them to nodes in the model. Most data sources allow for online discovery of data from the data source for populating the namespace. There is a limitation of 30 models per gateway, 500 nodes per model, and 8 nested node levels per model.

To stage tags

1. Perform one of these actions:

• Select Data Flow > Models.

• Select Data Flow and then under Organize, select Configure.

2. Select Edit next to a model.

3. Under Tags, select Stage Tags.

4. (optional) Filter the list of tags by data source or by type.

5. Select tags and then select Save.

Tags appear in the workspace.

6. To remove tags that are not being used by the model, from the workspace, select tags and then select Unstage Tags.

Tags are elements of information from your data source. After adding a model and staging tags, associate tags to elements (nodes and properties) in the model.

To bind a tag to a model

1. Perform one of the actions:

• Select Data Flow > Models.

• Select Data Flow and then under Organize, select Configure.

2. Select Edit next to a model.

3. (optional) Filter the list of tags.

4. Perform one of these actions:

• To select and bind multiple tags, under Tags, select the tags to associate to the node or property and then select Bind. The tags appear underneath the top-level node.

• Select a tag, select a node or a property and then select Bind.

• Drag and then drop a tag on a node or a property.

5. Select Save.

When FactoryTalk Smart Object instances are configured in a Studio 5000 Logix Designer controller, connecting to that controller by importing an .acd file replicates the FactoryTalk Smart Object model created in the controller.

Use case 1: Smart Objects [Add a data source from a Studio 5000 Logix Designer Project File (.acd)] + Logix Echo Controller (Emulate PAC)

Add data sources from offline Studio 5000 Logix Designer controller project files to populate a namespace and collect data from data tags and FactoryTalk Smart Object instances. The supported file type is “.acd”. FactoryTalk Smart Object instances are only available from the Studio 5000 Logix Designer project file (.acd) data source type and require an EtherNet/IP (CIP) driver. The data from the “.acd” project file is later accessed from the device or server running the Studio 5000 Logix Designer project file.

FactoryTalk Edge Gateway is compatible with Logix firmware revisions 16.00 or later. FactoryTalk Smart Object instances are supported in Logix firmware revisions 28.00 or later.

The Studio 5000 Logix Designer project file (.acd) data source is compatible with these drivers: FactoryTalk Live Data and EtherNet/IP (CIP).

To add a data source from a Studio 5000 Logix Designer project file (.acd)

1. Select Data Flow > Data Sources, and then from Configure Data Sources, add a data source.

• To add the first data source, select Add Data Sources.

• To add a subsequent data source, select Add .

2. Enter the name and then from Data Browse Path Type, select Logix Project File (ACD).

• The name must contain only letters, numbers, and hyphens.

3. Browse for the file path and select the .acd file.

4. (optional) To generate the namespace, select Auto-populate namespace.

5. From Driver for Collection, select the FactoryTalk Live Data or EtherNet/IP (CIP) driver.

6. Enter the collection path.

7. Select Save to add the data source.

Use Case 2: Add a data source from a FactoryTalk Live Data Shortcut – CompactLogix controller and VSD

Add data sources from FactoryTalk Live Data to populate a namespace and collect data from data tags. The data source is also a shortcut to these source types: HMI TagDb, OPC DA server, OPC UA server, and FactoryTalk® Linx™ shortcut.

The FactoryTalk Live Data Shortcut data source is compatible with the FactoryTalk Live Data driver.

To add a data source from a FactoryTalk Live Data shortcut

1. Select Data Flow > Data Sources, and from Configure Data Sources, add a data source.

• To add the first data source, select Add Data Sources.

• To add a subsequent data source, select Add.

2. Enter the name and then from Data Browse Path Type, select FactoryTalk Live Data Shortcut.

• The name must contain only letters, numbers, and hyphens.

3. From Driver for browsing, select the FactoryTalk Live Data driver.

4. Select a network or local path and enter the browse path.

5. (optional) To automatically generate the namespace, select Auto-populate namespace.

6. Perform one of these:

• Select Use browse path for data collection.

• From Driver for Collection, select the FactoryTalk Live Data driver for the data collection path and enter the collection path.

7. Select Save to add the data source.

FactoryTalk Edge Gateway applications

Applications in FactoryTalk Edge Gateway collect data from tags, models, or FactoryTalk Smart Object models and then send the data to the configured server or IoT device. For example, the ThingWorx application in FactoryTalk Edge Gateway sends data to the configured PTC ThingWorx Foundation server.

FactoryTalk Edge Gateway supports these applications:

• Azure IoT Hub

• MSSQL

• ThingWorx

• Test Client

• MQTT

In FactoryTalk Edge Gateway, user can monitor the values of live data flowing into the gateway.

Test Client Application

Use the Test Client application to test the connection to the gateway and make sure that your data sources and models are properly configured and sending the correct data. The advantage of the Test Client application is that you do not need to configure an application for a server or IoT device to test the connection. The Test Client application displays live data streaming into the gateway when Enable Data Flow is selected.

ThingWorx Application

For users that have IIoT (Industrial Internet of Things) applications or solutions using PTC ThingWorx, FactoryTalk Edge Gateway provides a method to collect information from Rockwell Automation and third-party sources and send it to a ThingWorx Foundation server.

FactoryTalk Edge Gateway facilitates creating objects that are required in ThingWorx to represent the model, which reduces the time for deploying a ThingWorx application. To communicate with the ThingWorx Foundation server, add a ThingWorx application in FactoryTalk Edge Gateway, and then configure and deploy the application. During configuration of a ThingWorx application, models selected from the Models dialog box appear in Preview as a hierarchical structure of ThingWorx Things.

To add a ThingWorx application:

1. In the top-right corner, select Online to go online with the gateway. FactoryTalk Edge Gateway displays a green banner: Gateway Connection: Online.

2. Select Data Flow > Applications.

• To add the first application, select Add Application.

• To add a subsequent application, select Add .

3. From Select Application, select an application

4. Select Next.

5. Set these parameters:

• Application Name. Defines the application name.

• ThingWorx Server Host Name. Defines the name or IP Address of the server host.

• ThingWorx Server Port. Indicates the communication port between the <TXW> Foundation server and the gateway.

• ThingWorx Server Application Key. Defines the security token used for authentication with the <TXW> Foundation server.

• ThingWorx Logging Level. Set the logging level for the ThingWorx application to one of these options:

• Info.This level logs important system information. Exceptions are not logged.

• Trace. This level is used to help debug issues while developing an application. Trace logs provide detailed information to troubleshoot issues. This is the most detailed log.

• Debug. This level is used to help debug issues. The log contains information about what the code is doing and the values of important variables.

• Warn. This level logs unexpected exceptions that are acceptable or can be worked around. It includes problems that could become errors if left unattended.

• Error. This level logs issues that affect the usage or performance of the system.

• Force. This level logs issues that affect the usage or performance of the system. This option makes sure that all errors are logged.

• Audit. This level logs messages in a separate audit file.

• Store and Forward. This setting enables storing the data entering the application in a database on the gateway. This setting also functions as a buffer for data entering the store database at higher speeds than it is forwarding to the application.

SQL Server Application

For users that have IIoT (Industrial Internet of Things) applications or solutions using SQL Server, FactoryTalk Edge Gateway provides a method to collect information from Rockwell Automation and third-party sources and send it to a SQL Server.

FactoryTalk Edge Gateway facilitates creating objects that are required in SQL Server to represent the model, which reduces the time for deploying a SQL Server application. To communicate with the SQL Server, add a SQL Server application in FactoryTalk Edge Gateway, and configure and deploy the application. During configuration of a SQL Server application, models selected from the Models dialog box appear in Preview as a hierarchical structure of SQL Server. Before you begin, create a username for the database and set permissions using the user interface in SQL Server Management Studio.

To add a SQL Server application:

1. In the top-right corner, select Online to go online with the gateway. FactoryTalk Edge Gateway displays a green banner: Gateway Connection: Online.

2. Select Data Flow > Applications.

• To add the first application, select Add Application.

• To add a subsequent application, select Add .

3. From Select Application, select an application

4. Select Next.

5. Set these parameters:

• Application Name. Defines the application name.

• Server Name. Defines the name or IP Address of the server host.

• SQL Server Instance Name. Enter the name of the SQL Server instance.

• The SQL Server Browser service must be running anytime the SQL Server instance name is the connection method. If you enter information in both SQL Port and SQL Server Instance Name, FactoryTalk Edge Gateway uses the SQL Server instance name. To configure and debug the SQL Server, see Microsoft SQL Server technical documentation.

• SQL Port. Enter a port number for SQL Server if not using the standard 1433 port number.

• Database Name. Indicates the communication port between the SQL Server and the gateway.

• Table Name. Enter the name of a table already in use by FactoryTalk Edge Gateway or enter the name of a new table. If the table does not exist, FactoryTalk Edge Gateway creates the table. Only use tables that FactoryTalk Edge Gateway creates.

• Domain. Enter domain or workgroup name to connect to SQL Server by using Windows authentication. Leaving this field blank uses SQL Server authentication.

• Username. Enter the specific username for the database. Recommended practice is to create a username with the correct access and permissions for the database to use with the FactoryTalk Edge Gateway SQL Server endpoint.

• Password. Enter the password associated with the username.

• Encrypt. Encrypt the connection to the SQL server.

• Enable Fire Insert Trigger. Enable the execution of database triggers on bulk insert of data to SQL table from FactoryTalk Edge Gateway.

• Store and Forward. Enables the storage of data entering the application in a database on the gateway. This setting also functions as a buffer for data entering the store database at higher speeds than the data is forwarded to the application. At each configured Fetch Interval, a maximum amount of data fetched from the store database is 256 KB.

Select Store and Forward to enable the Maximum store size (bytes) and Fetch Interval (ms) settings. In case connectivity is lost to the external application, having Store and Forward enabled prevents periods of lost data to SQL Server. Store and Forward stores data in the database until the connection is restored.

• Aggressive Database Cleaning. Select to delete 50% of the oldest data from the store when maximum store size is exceeded.

• Maximum store size (bytes). This setting defines the maximum size of the store database on the gateway, in bytes. When the maximum store size is reached, the oldest data is deleted to make space for new application data. Select Store and Forward to enable this setting.

• Fetch Interval (ms). This setting defines the interval of time to egress (that is, forward) data from the store database, in milliseconds. Data continues to be egressed to the ThingWorx application until the store database is empty. When egress fails, the data remains in the store database. Select Store and Forward to enable this setting.

Set database permissions for SQL Server:

To set database permissions in SQL Server Management Studio

1. Open SQL Server Management Studio.

2. In the Server tree in the Object Explorer, right-click Logins and select New Login.

3. Create a username by using any name or password.

4. Assign the Server role bulkadmin.

5. In the Databases tree, right-click Security > Users.

6. Select the newly created username and assign these roles:

• db_datareader

• db_datawriter

• ddadmin

7. Exit SQL Server Management Studio.

Azure IoT Hub Application

For users that have IoT (Industrial Internet of Things) applications or solutions using Microsoft Azure IoT Hub, FactoryTalk Edge Gateway provides a method to collect information from Rockwell Automation and third-party sources and send it to a Microsoft Azure IoT Hub.

FactoryTalk Edge Gateway facilitates creating objects that are required in SQL Server to represent the model, which reduces the time for deploying a SQL Server application.

To communicate with the Azure IoT Hub, add an Azure IoT Hub application in FactoryTalk Edge Gateway, and configure and deploy the application. During configuration of an Azure IoT Hub application, models selected from the Models dialog box appear in Preview as a hierarchical structure of Azure IoT Hub.

To add an Azure IoT Hub application

1. In the top-right corner, select Online to go online with the gateway. FactoryTalk Edge Gateway displays a green banner: Gateway Connection: Online.

2. Select Data Flow > Applications.

• To add the first application, select Add Application.

• To add a subsequent application, select Add.

3. From Select Application, select an application

4. Select Next.

5. For Azure IoT Hub, set these parameters:

• Application Name. Defines the application name.

• Connect to Edge Runtime. Do not select for regular IoT applications.

• Use WebSockets. Select to use MQTTWS protocol. Uses port 443.

• IoT Device Name. Enter the name of the IoT device created in the Azure portal.

• IoT Device Connection String. Enter the Primary/Secondary Device connection string from IoT Hub device.

• Store and Forward. Enables the data storage in a database on the gateway. This setting also functions as a buffer for data entering the store database at higher speeds than the data is forwarded to the application.

• Maximum store size (bytes). Defines the maximum size of the store database on the gateway, in bytes. When the maximum store size is reached, the oldest data is deleted to make space for new application data.

Select Store and Forward to enable this setting.

• Fetch Interval (ms). Defines the interval of time to egress (that is, forward) data from the store database, in milliseconds. Data continues to be egressed to the Azure application until the store database is empty. When egress fails, the data remains in the store database.

Select Store and Forward to enable this setting.

6. Select Save.

Back up FactoryTalk Edge Gateway configuration

You can create a manual backup of your FactoryTalk Edge Gateway configuration.

To back up a FactoryTalk Edge Gateway configuration

1. Launch the configuration tool.

2. If prompted, upload the configuration from the gateway to resolve differences.

3. Exit the configuration tool.

4. Browse to the location of your resources directory. For example, C:\Program Files (x86)\Rockwell Software\FactoryTalk EdgC:\Program Files (x86)\Rockwell Software\FactoryTalk Edge Gateway\fteg-ct.

5. Copy the resources directory from that location.

6. Browse to your backup location. For example, C:\fteg-ct\backup\resources.

7. Save the resources directory.

Restore FactoryTalk Edge Gateway configuration

If you created a backup of the resources directory, then you can restore your FactoryTalk Edge Gateway configuration from the backup.

To restore a FactoryTalk Edge Gateway configuration

1. Exit the configuration tool if running.

2. Browse to the resource directory location you created in the backup procedure. For example, C:\fteg-ct\backup\resources.

3. Copy the resources directory from that location.

4. Browse to the location where you want to restore the resources directory. For example, C:\Program Files (x86)\Rockwell Software\FactoryTalk Edge Gateway\fteg-ct\resources.

5. Paste the resources directory to C:\Program Files (x86)\Rockwell Software\FactoryTalk Edge Gateway\fteg-ct\resources.

6. Start the configuration tool.

7. From the menu, select Online to go online with the gateway.

8. When prompted, download the configuration to the gateway.